Run Whitehall Scenario 1 model

Create a directory c:\RunModel

This model was constructed with Groundwater Vistas 7.23 build 1. If you have GW Vistas GUI you can open the modeling file WH_Scenario1.gwv to check the model parameters except river package (RIV), evapotranspiration package (EVT), and wells (WEL). Don’t run the model with this gwv file.  Instead, use the MF2005win32 interface (Scenario1) to run the model with MODFLOW input files (listed in the directory). The MF2005win32 interface is ready to run - in transient mode. To run in another directory, copy all listed input files to the new directory, then edit the path for the input files (input tab) to match the new directory. Similarly, edit the path for output files (output tab), e.g. the heads file (*.hds).

Note:

Make sure to replace the WEL, EVT, and RIV packages input files with the following:

Scenario1_Wells.wel for Well package (WEL)

Scenario1_Evt.evt for Evapotranspiration package (EVT)

Scenario1_River.riv for River package (RIV)

Also make sure to include the initial heads file WH_SS_ini_heads.hds (unit 15)

Run the model with DOS version of MODFLOW2005

Download the USGS MODFLOW2005 from MODFLOW-2005 Downloads (usgs.gov).

Extract from the zip file.  Copy the executable file mf2005 from the Bin folder to the model directory (Run Model). Run mf2005 with the model name file Scenario1_DOS.nam

- Use Groundwater Vistas file WH_Scenario1.gwv to retrieve output and display results